Executive Summary

NYC QA Lab — In-Basket Bag Hook Retrofit

Date: July 27, 2026  |  Scope: 7 NYC QA Caper Carts

PURPOSE

This document summarizes the results of the in-basket bag hook retrofit completed across all 7 Caper Carts in the NYC QA Lab. The retrofit involved installing bag hooks, rebooting/reinitializing carts, and validating scale accuracy through weight testing.

STATUS: COMPLETE ✓

All 7 NYC QA carts have successfully completed the bag hook retrofit. Every cart passed required weight checks following bag hook installation and reinitialization. Detailed test results and cart-level data are documented in the supporting Excel file.

KEY FINDINGS

1.  All 7 carts passed weight checks

Every cart passed the required weight validation after bag hook installation and reinitialization. No recalibration was needed post-reinitialization.

2.  Reinitialization (not just reboot) is required

A detailed comparison was run on cart CAM3STMI011MI23D0250 across three scenarios:

| Scenario | Max Weight (80 lb) Test | Outcome |
| --- | --- | --- |
| Before bag hook installation | Baseline | Passed |
| After bag hook install — reboot only | Failed | Reinitialization required |
| After bag hook install — reinitialization | Passed | No recalibration needed |

OPEN QUESTION — Bag Weight Standard

Testing used an approximate recommended bag weight of 2.75 lbs per hook. However, the hooks have physical capacity to hold more bags. If retailer staff adds bags above this threshold, the cart may fail the max weight (80 lb) test due to the additional tare weight exceeding the reinitialized scale baseline.

Viz: Andrew Cha, Dan Chen

RECOMMENDATION

Establish a clear standard for bag quantity and weight per cart hook, covering:

•  Maximum bag weight per hook (with appropriate margin below the 80 lb test threshold)

•  Handling guidance for heavier bag types (e.g., paper vs. plastic)

•  Retailer instructions to prevent overstocking bags on hooks

•  Field SOP update to require reinitialization (not just reboot) after any bag hook installation

ACKNOWLEDGEMENTS

•  John Poli & QC Team — facility access and on-site support

•  Rehnuma Tarannum — retrofit process and testing support

•  Veronica Yang — parts delivery coordination

•  Andrew Cha — overall coordination

SUPPORTING ASSETS

•  Excel file: NYC QA lab test results + cart-level detail data

•  Photos / Videos (for Retail Go-To-Market enablement plan, attn: Andrea Metcalfe):

drive.google.com/drive/u/0/folders/1xF3u6jE8hbWQ8ac8hdQttkyjf3GKXWU9

•  All 12 requested enablement plan sections completed (minus sections previously discussed as exceptions)