Caper Cart Weight Check SOP

Operations Procedure for Verifying Caper Cart Scale Accuracy

| Document | Owner | Version | Effective Date |
| --- | --- | --- | --- |
| Caper Cart Weight Check SOP | Operations | 1.0 | May 6, 2026 |

# 1. Purpose

This SOP defines the operational procedure for verifying Caper cart scale accuracy in the field.

# 2. Scope

Applies to all Caper carts deployed at retailer locations. Use this procedure during weight checks, after cart re-initialization, after TU swaps, and any time scale accuracy is in question.

# 3. Materials Required

- Calibration checks — NIST-traceable weights: 5 lb, 20 lb, 25 lb, 40 lb, 80 lb

Highlighted values indicate NIST-traceable calibration weights.

- Caper cart, charged to at least: 25–30%

- Sample for RSA tracker (Excel) — one row per cart tested

- Flat, solid, level surface free from drafts (no rugs, no fans, no automatic doors nearby)

# 4. Tolerances

Apply Maintenance Tolerance for carts already in the field.

| Test Weight (lbs) | Maintenance Tolerance* (carts in field) | Acceptance Tolerance** (new deployments) |
| --- | --- | --- |
| 0 – 5.0 | ± 0.01 lb | Must read exactly 0.00 – 5.00 lb |
| 5.01 – 20.0 | ± 0.02 lb | ± 0.01 lb |
| 20.01 – 40.0 | ± 0.03 lb | ± 0.01 lb |
| 40.1 – 80.0 | ± 0.05 lb | ± 0.02 lb |

*Maintenance Tolerance applies to equipment in actual use.

**Acceptance Tolerance applies to first-time deployment

# 5. Pre-Check Setup

- Confirm the cart battery is at least 25–30% charged.

- Unstack the cart and disconnect it from any other cart.

- Move the cart to a flat, solid, level surface — no rugs, no uneven flooring.

- Move the cart away from any source of air movement: HVAC vents, fans, automatic doors.

- Empty the basket completely; verify it is clean and in good condition.

- Visually confirm the basket is aligned, using the front cameras and the back plate as reference.

# 6. Open the Weight Check Screen

Use the Shopping flow to expose live weight readings on the cart display.

- Press Start Shopping — the cart navigates to the shopping home page.

- Go to the Shopping screen and use “Add Item by Weight” to expose the live weight reading.

⚠ Caution: Place every weight in the center of the basket unless the procedure explicitly says otherwise. Place weights gently — do not drop them. Wait for the reading to settle (about 2 seconds) before recording.

# 7. Increasing Load Test

Add weights cumulatively in the center of the basket. After each step, wait for the reading to stabilize, then record the value in the corresponding column of the tracker (D through I). The cart must read within the applicable band at every step.

| Step | Action | Test Weight (lb) | Maintenance Tolerance (lb) | Acceptance Tolerance (lb) |
| --- | --- | --- | --- | --- |
| 1 | Weigh 5 lb in Center | 5.00 | 4.99 – 5.01 | Must read exactly 5.00 |
| 2 | Weigh 20 lb in Center | 20.00 | 19.98 – 20.02 | 19.99 – 20.01 |
| 3 | Weigh 40 lb in Center | 40.00 | 39.97 – 40.03 | 39.99 – 40.01 |
| 4 | Weigh 80 lb in Center | 80.00 | 79.95 – 80.05 | 79.98 – 80.02 |
| 5 | Weigh 20 lb in Center | 20.00 | 19.98 – 20.02 | 19.99 – 20.01 |
| 6 | Weigh 5 lb in Center | 5.00 | 4.99 – 5.01 | Must read exactly 5.00 |

If any step fails — stop the increasing load test, remove all weights, and retry once more. If it still fails, mark the cart as failed and continue to the Shift Test for diagnostic completeness.

# 8. Shift Test

Remove all weights from the basket. Place a single 25 lb. weight (Norm is 35% of max load (80 lbs.) = 28 lbs.) (Closest standard weight is 25 lbs.) in each of the four basket quadrants in the order shown below. Wait for the reading to settle, record the value in the tracker (columns J through M), then move the weight to the next position. Use the four-corner pattern from Figure 1 of the regulatory SOP — do not use the diagonal pattern from Figure 2.

| Step | Position | Test Weight | Maintenance Tolerance (lb) | Acceptance Tolerance (lb) |
| --- | --- | --- | --- | --- |
| 1 | Bottom Left | 25 lb | 24.97 – 25.03 | 24.99 – 25.01 |
| 2 | Bottom Right | 25 lb | 24.97 – 25.03 | 24.99 – 25.01 |
| 3 | Top Right | 25 lb | 24.97 – 25.03 | 24.99 – 25.01 |
| 4 | Top Left | 25 lb | 24.97 – 25.03 | 24.99 – 25.01 |

Basket Position Diagram (top-down view)

| Top Left (Step 4) | Top Right (Step 3) |
| --- | --- |
| Bottom Left (Step 1) | Bottom Right (Step 2) |

(Text representation of the original figure — four-corner pattern, weight centered in each quadrant.)

# 9. Recording Results in the Tracker

Open the Sample for RSA Excel tracker and add one row per cart tested. The tolerance reference table at the top of the sheet is read-only; add new entries starting at the first empty row beneath the existing data.

| Col | Field | What to Enter |
| --- | --- | --- |
| A | Test Date | Date the check was performed (e.g., 5/6/2026) |
| B | Retailer Name | Name of the retailer location |
| C | Cart No | Cart number printed on the cart label |
| D | Increasing Load Test – 5 lbs | Reading after Step 1 (target 5.00) |
| E | Increasing Load Test – 20 lbs | Reading after Step 2 (target 20.00) |
| F | Increasing Load Test – 40 lbs | Reading after Step 3 (target 40.00) |
| G | Increasing Load Test – 80 lbs | Reading after Step 4 (target 80.00) |
| H | Increasing Load Test – 20 lbs | Reading after Step 5 (target 20.00 on offload) |
| I | Increasing Load Test – 5 lbs | Reading after Step 6 (target 5.00 on offload) |
| J | Shift Test – Bottom Left 25 lbs | Reading with 25 lb in Bottom Left position |
| K | Shift Test – Bottom Right 25 lbs | Reading with 25 lb in Bottom Right position |
| L | Shift Test – Top Right 25 lbs | Reading with 25 lb in Top Right position |
| M | Shift Test – Top Left 25 lbs | Reading with 25 lb in Top Left position |
| N | Inspection Passed (Y/N) | Y if all readings within tolerance, otherwise N |
| O | Comments / Notes | Reason for failure, anomalies, or observations |

## 9.1 Pass / Fail Decision

Compare each recorded reading against the band you applied (Maintenance for in-field carts, Acceptance for new deployments). The cart passes only if every reading — increasing load and shift — is inside the applicable band.

| Result | Action |
| --- | --- |
| PASS — every reading within the applicable band | Mark column N as “Y”. Cart returns to service. |
| FAIL — one or more readings outside the band | Mark column N as “N”. Add reason in column O. Pull cart from service, re-initialize, and retest. Escalate to the Caper Operations lead if the failure repeats. |

## 9.2 Sample Row (for reference)

Below is the existing sample failure row from the tracker. Cells highlighted in orange are the readings that fell outside the Maintenance Tolerance band, which is why column N is “N” and column O is annotated.

| Date | Retailer | Cart | 5 lb | 20 lb | 40 lb | 80 lb | 20 lb | 5 lb | BL 25 | BR 25 | TR 25 | TL 25 | Pass? | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4/23/26 | Store | 4 | 5.00 | 19.97 | 39.94 | 79.90 | 20.00 | 5.00 | 24.94 | 24.93 | 25.00 | 24.99 | N | Sample failure |

# 10. Handling a Failed Check

- Pull the failed cart from service immediately and inform the POC to put the cart on Coffee Break Screen

- Re-run and re-check the cart. Retest once more.

- If the cart fails the retest, Pull the failed cart from service immediately, place it on the furthest position of the fixed charger away from customers so it remains charging but visually separated, and inform the POC to put the cart on Coffee Break Screen.

- Escalate to the Caper Operations lead with the tracker row and a photo of the cart number.

# 11. References

- Caper Weights and Measures Test SOP for Operations (regulatory parent document)

- NIST Handbook 44, paragraph G-T.1 — acceptance and maintenance tolerances

- NIST Handbook 44, paragraph T.N.4.4 — shift / section test interpretation

- Sample for RSA.xlsx — weight check tracker

# 12. Revision History

| Version | Date | Author | Change |
| --- | --- | --- | --- |
| 1.0 | May 6, 2026 | Operations | Initial release for field weight checks. |